Laser Burglar Alarm

- Objective:

The objective of this project is to generate an efficient alarm system using LDR and a laser beam.

The other main objective is to generate a working PCB design and layout by using the Ki Cad/Altium designing tool.

- Approach:

- Breadboard Testing:

The components are connected as per the circuit diagram. The buzzer is setoff is there is no laser beam falling on it. This proves that the 2N222 acts as a switch and when the circuit is complete the alarm beeps and if not that is when laser beam is falling on the LDR the alarm does not beep.

2.PCB Design

Once the design is tested on the breadboard it is etched on the PCB.

The components are then soldered along with the tracks and it is tested using a 9V battery and buzzer.

- EDA Technology used:

The schematic and PCB tracks were routed using altium design tool.

Altium is a special software used for designing a PCB and it is easy to use.

- Schematic Work:

- Simulation Results:

- When the laser beam is not pointing towards the LDR the buzzer goes off.

- When the laser beam is pointing towards the LDR the buzzer is not making any noise i.e the circuit is complete

- Bill of Materials (BOM) Generated:

| Components | Cost(Rs.) | Quantity |  | Total(rs.) |
| --- | --- | --- | --- | --- |
| 2n222 | 15 | 1 |  | 15 |
| Resistor | 1 | 1 |  | 1 |
| LDR | 5 | 1 |  | 5 |
| Buzzer | 10 | 1 |  | 10 |
| Battery | 10 | 1 |  | 10 |
| PCB | 30 | 1 |  | 30 |
| Total |  |  |  | 76 |

- PCB layout:

- References/ Materials /Books used:

1.http://www.slideshare.net/lasersystems

2. www.instructables.com/id/Extremely-simple-laser-alarm.

3. www.electroschematics.com/4929/laser-based-door-alarm

- Conclusion:

- By using a line laser and LDR we were able to make a simple laser circuit that detects objects passing through it.

- Laser technology products will calculate distance by measuring the time of flight of very short infrared light.

- It has a wide number of applications some of them are mentioned below.

- Application: